Region 9 Army MARS Training Topics for “phone bridge” use
TRAINING TOPIC   –  Using Other Processes of Station Master
LESSON INTENT –  To overview the some of the functions of Station Master including QRU and ZEE.
REFERENCES – ACP-125,Station Manager Machine Functions  
EXPECTED TIME TO DELIVER – 60 Min.
CONTENT OUTLINE & KEY POINTS – Understanding some of the functions of Station Master.

1. NCS form – Net Roster, Turnover Roster, Traffic Sum, and Import Roster

2. Load last RX, to SP, CT Message Que Pull Down.

3. Configuring Station Manager to enable the QRU, and ZEE process.

4. Going over the arguments to be inserted in the Order Wire Command line to filter the results to your requests.

5. Giving an actual demonstration of the complete process.

Overview
1. Some of the commonly used functions of Station Manager

· NCS Form

· Function Tabs and Pull Downs

2. The machine functions implemented in Station Manager are a subset of the ones available in the
Message Machine. The operator may query a list of available messages, and may then choose which
message to request from the offering station.
Terminal Controls, requesting station.
These controls in the Station Manager Terminal window allow the operator to query another station for
a list of available message files.
• QRU – a signal to request a list of available messages.
• ZEE – a signal to request a specific message be transmitted.
• QSL – a proactive way to inform the sending station that the message was received by you.
• Order Wire Line Edit – query or message name parameter is entered here for sending to the

other station. This is a dual use field, and it is also used for normal order wire short messages.


The Process
I wish to query another station for available messages (QRU). I may do one of three things in the QRU process.

a. Leave the Query Line Edit field empty to get a list of messages available for my call sign.


b. Enter a full or partial Routing Indicator or call sign to do a wildcard match for pertinent


messages(I.E. UHEACVA,UHWAACO OR UH).
Finally, after choosing my method from above, I will select a call sign from the Heard List or
click the SELCAL radio button and enter a destination call sign of the other station, and finally,
click the QRU button.
The offering station will respond to a QRU request with a list of messages if there are any.
AAR3ABC DE AAR3MP-CM
QTC4 UH
UHEACVA_UHEACVA_0025_2411811
UHEACVA_UHEACVA_0032_2550133
UHWAACO_UHEACVA_0020_2381842
UHWAACO_UHEACVA_0028_2451541
K
If I decide that I wish to get message number 0028 from that list, the easiest way to do so is to
copy the full message name with a double-click and Ctrl-C and paste it (Ctrl-V) into the Query
Line Edit field, then click the ZEE button. A pre-formatted request will be sent to the offering station and that message file will be transmitted to me.  If I choose to, and it is not required, I may then click the QSL button to send a pre-formatted message to the offering station indicating that I have “rogered” for that specific message. That QSL message will be saved with a special “QSL_QSL…..txt” file name for quick visibility.
Query Details
The wildcard match on the filenames is flexible.  Thus, a query such
as: QRU UHEA Will match any message files offered which begin with UHEA
QRU UHEACVA_UHEACPA Will match any messages offered TO VA tributary from PA tributary, etc.
(UHEACVA_UHEACPA.txt).

The wildcard match can also be used with mixed RI and destination call.  Thus, a query such as: QRU UH,AA,AFA will match any files offered which begin with UH, AA,AFA

QRU UH,AA,AFA will match any files offered TO  UHCWBC UHCVBC UHCHBC, AAR9WF, OR AFA9DU
Station Manager Filename Format
The Station Manager filename format is the Destination call or RI, followed by the Source call or RI,
the message number and Julian date/time group (parts of FL2 and FL3).
Example: A message originated by a tributary station in Pennsylvania to be routed to a staff officer in
Virginia would be something like: UHEHCVA_UHEACPA_0023_2451234.txt
Station Manager’s Message Editor as well as the Scratchpad help you to name these files appropriately.
Files arriving over the radio which are successfully processed by Station Manager will automatically
have this filename format if possible based on the content of the message. Other sources of messages
such as email or poorly formatted messages will be the responsibility of the station to repair the message internally if appropriate, or to at the very least provide the appropriate filename in the above format.
How to Offer Messages Within Your Station Manager
To offer a message or messages for others to transfer, simply open the ACP message in the Scratchpad
and review it for accuracy of ACP format. If the format is correct and includes FL2 and FL3, it is very
likely that the button marked “To QRU” will successfully copy that text to a file in the QRU folder of
the Station Manager directory structure. The QRU folder is the landing spot for all messages offered to others.

If you are originating a message for this purpose, simply use the Station Manager Message Editor and
after previewing your completed ACP message, click the “To QRU” button from the Actions area. The
completed message file will be written to the QRU folder.  Messages which have arrived over the radio may be MOVED to the QRU folder if they require relay.  Find the message in the  RECEIVED_CT folder using the Message Viewer in the Terminal. Right click on the filename and choose “Move to QRU”. Ensure the filename format is correct.

How can I see what is in my QRU folder?”
To view the contents of the QRU folder, simply click the Message Viewer tab in the Terminal window and navigate to the QRU item in the folder list. A list of the files in QRU will be displayed below that, and you may click on each one to preview the contents in the right-hand file viewer ar

[image: image1.jpg]W TK-90 - tation Manager - ver. Mar 30 2017 - This System IsFor Processing Up To UNCLASSIFIED/FOUO.
File Open Actions Help
(600~ Resetioden | Abortx

Terminal | Message Viewer

[zcumarie - Jocumese -2 -]

CIPHER TEXT MODE



[image: image2.png]W TK-90 - tation Manager - ver. Mar 30 2017 - This System Is For Processing Up To UNCLASSIFIED/FOUO. - o x
Fle Open Actions Help
ResetModem | AbortTX

Terminal | Message Viewer | Config

[aaay ARASR7-CM DE AARSCB

QRU UH,AA
X
Incoming Transmission...wait

[16:22:55]

Checksum Passed
Received SOT

FM: AAASRT >> TO: ARRSCB

AARSCB DE ARASRT-CM

QTC 3 UH,AA
AARSTC_087_EEEE_PASS
UEWDBCA_UEWHEC_0043_2300100
VXXX UEWABAZ_0024_1442330
AR





[image: image3.jpg]W TK-90 - Station Manager - ver. Mar 30 2017 - This System Is For Processing Up To UNCLASSIFIED/FOUO. =m0
File Open Actions Help

0L v | ResetModem | AbortTX

P |

UHOX_UHWABAZ_0024

=n o





AAA9R7-CM DE AAR9CB

ZEE AAR9TC_087

K
Incoming Transmission...wait
[23:29:00]
Checksum Passed
Received ACP Message
Received SOT
FM: AAA9R7 >> TO: AAR9CB
VZCZCMMM087     

A9A DE AAR9TC NR 087 

ZNY EEEEE

R 252330Z May 2017

FM AAR9TC

TO A9A

BT

UNCLAS SVC

TIME: 2330Z  NET: A9A

NCS: 

AAR9TC 

AFA9WD 

AAR9CB 

AFA9VW

AAR9HH 

AAR9CV 

AAR9CG 

AAR9CT 

AAR9EV 

AAR9WF

AAR9KS 

AFA9DU 

AAM9S 

AAR9UC 

AAR9OZ 

AFA9XY 

AFA9CH 

AAR9KE/T 

BT

NNNN
Stored ACP Message:
AAR9TC_087_EEEE_PASS.txt
[image: image4.png]W TK-90 - tation Manager - ver. Mar 30 2017 - This System Is For Processing Up To UNCLASSIFIED/FOUO. - o x
Fle Open Actions Help
600 - | ResetModem  AbortTX

a1

Incoming Transmission...wait

AAASRT

[23:46:51]
Checksum Passed
Received SOT

FM: ARASRT >> TO: ARRSCB

AARSCB-CM DE AAASRT
QSL UHXAAA_UEWHECP_0224
AR

Stored Message:
QSL_QSL_20170831_234651_PASS_ZYG.txt






PRACTICAL EXERCISE 

On Radio Net or between Individual Stations.
US Army MARS Region 9
Training Topic – 22 MAY 2018 – Page 8
Prepared by AAA9R7

